- Different Rank-Indicator Performance

CDL-A, rank 1 2 3 4 and flexible:

- At low SNR, forcing 3 and 4 layers perform better than flexible and 1,2 layers

- At high SNR, the performance makes sense. 
Flexible> force 2>force 3>force 1 >force 4 （Should be like this no matter what SNR?）. The flex case perf. match with L1, 2 When >23dB, seeing impact of TXEVM (by basestation), 3.5% error clipping signal ~23dB (1dB compression) diverging point – when comparing to real world case

| Test\dB | -5 | 10 | 30 |
| --- | --- | --- | --- |
| Flex. | 16QAM (0.6) QPSK (0.4) [1.5] | 64QAM (0.9) 16QAM (0.1) [1.8] ? too optimistic | 64QAM [2.1] |
| F1 | 16QAM (0.3) QPSK (0.7) | 64QAM | 64QAM |
| F2 | QPSK | 16QAM (0.3) 64QAM (0.6) | 64QAM |
| F3 | QPSK | 16QAM (0.55) QPSK (0.45) | 16QAM (0.3) 64QAM (0.7) |
| F4 | QPSK | QPSK | QPSK (0.45) 16QAM (0.65) |

101-4 single layer vs 2 layers for TDL channel model under different config

Now for CDL-C model:

Possible reasons for poor performances at low SNR:

- CQI, RI from CSI feedback is simply not accurate? Compare with perfect channel estimation. TODO

**Not my results**

Maybe TDL/CDL have different gap size? Maybe certain models lead to larger gaps?

- How CDL-C fix rank 1 table 1 vs table 2 performs.

CDL-C, fix to layer 1, table 1 vs table 2:

Bottleneck for table 1, doing better for table 2. E.g. Shannon limit? (theoretical limit)

- Maybe have a look at the (received) constellation diagram?

- Only Implemented the function, no experiment has been conducted yet

- Might be able the explain some weird behaviors during the simulation

Phase noise? Not AWGN model. -> need to explain during report, maybe this is the real system performance.

No 3GPP requirements on this…