Topics investigated during the past two week:

- CSI, channels in detail

- Water filling

- 3GPP’s progress

- remote computation

  - now able to submit timely computing tasks remotely to the teaching system at CUED

Technical

- How is it possible to achieve 4 layers using type 1 codebook? Shouldn’t the maximum (limited by CSI) rank be 2?

Or is it saying: gNB has 8 layers to 4UE x 2 layers using Type 1 codebook, but in the simulation I though there is no such thing as multi-user?

X 2 LDPC codewords for all the codeword。 8 Layers max for all

- Why use PMI random instead of follow during simulation?

  - PMI follow vs. random. Comparison to get the premium due to pmi？

- For the csi: cri->RI->PMI->CQI->LI chain, will the water-filling algorithm be implemented? Or is the power allocation for each layer assumed to be same?

  - Theoretical waterfilling (per layer), achieve by LDPC design SINR (per 4 layers - codeword) different (layer 1-4 must have the same modulation and coding scheme by 3g--) [512/1024 512/1024] [64QAM 16QAM]

    - MCS 13: exact same, manually fixed

    - Throughput higher at codeword 1, same SNR for both codewords (by different #MCS)

    - Technical specification

- Parameters such as [M N P Mg Ng], PI-PMI-CQI/AI CSI/Perfect CSI etc. somehow worth exploring?

  - [M N P Mg Ng] does not make that much difference

  - Not worth looking into (small improvement)

3GPP standard related

- Focus on TSG – RAN 4 (radio performance and conformance)? Yes

- Are there any summarizing presentation slides/reports that introduce what they are currently doing from Rel-19 -> Rel-20? (particularly related to my RQ)

  - Status report: spatial channel model for demodulation rel 19

RP-252927

  - Rel-14 TR 38.900, TR 38.901 SCM(E) leading to CDL, TDL-> Rel-15 decided to use simplified TDL model, hard to align simulation results using 38.901 CDL -> Currently all performance requirements are all based on TDL (ULA/X-pol), hard for RAN4 to show benefits for Rel-16 NR_eMIMO, Rel-17 NR_feMIMO, spatial components need to be introduced.

    - Antenna virtulization

    - CDL alignment (38.753)

    - Frequency generalization

    - Get ready for 6G (what channel model to use: CDL or TDL not yet decided)

    - CDL 4

  - What exactly is the role of RAN4? E.g. is the feature Rel-16 NR_eMIMO implemented

    - Measure perf. Under specific testing condition (SPATIAL FEATURE)

    - Cannot find abs. performance

    - Type 1 codebook conf. index 12345 in TDL no difference

    - Spatial design new features: cannot show in TDL at all

- Provide guidance to future work and final destination?

Miscellaneous:

- Teams Chat

- Cambridge face to face

- Second back-up timeslot?

What to do this & the next week:

- Further research topics

- Further Matlab simulations

- Others

- Where to go:

  - LDPC?

  - Massive MIMO performance characteristics

  - CSI?

  - Multi cluster TDL (new ones, self-construct)

  - SU-MIMO -> Multi-user MIMO