- Impact of CSI Feedback Granularity and PMI Codebook Size on Downlink MIMO Throughput in 5G NR

Find from demodulation requirement 38.101-4 (now for TDL) as benchmark

Key variables to explore:

- simParameters.CSIReportConfig.CodebookMode = [1, 2]; Single/Multi panel

- simParameters.CSIReportConfig.PMIMode      = {'Wideband', 'Subband'};

- simParameters.CSIReportConfig.SubbandSize  = [4, 8, 16, 32];

- CSI Periodicity = [2 5 10];

Scenarios:

- Velocity: [walk, drive, highway] -> Doppler shift

- MIMO layers: [2, 4, 8]

- Channel: CDL-C

Objective:

- Demonstrate the trade-off between cost of CSI-feedback with throughput under different scenarios, finding the optimal set of solutions

- How much extra codebook resolution or more frequent CSI reports are worth in terms of throughput?

- Provide design guidelines that link channel conditions and system targets to suitable CSI configurations, support 3GPP standards

Results to collect:

- Throughput % - SNR

- HARQ retransmission rate

- RI, layer utilization

Expectation:

- dependence of performance on CSI periodicity at higher velocities

- throughput gaps between coarse and fine codebooks, especially at high SNR and for multi layer transmission

- Diminishing marginal returns vs. increase in feedback cost

- Factor: Avg. Throughput over time period, if e.g. 1000 slots, avg them, how many slots are `lost’

  - % throughput decrease vs. abs. (max) throughput loss

  - Maximum optimized throughput for a given channel: optimal throughput (so at high SNR if csi on, always <100%, if csi off can reach 100%) -> optimized

    - 3D plot (given channel, snr x, throughput z, csi one variable to investigate (hope there to be a max.)

      - Roll-off (skewed or not?) -> decide on what to use in future

- Power: avg. due to csi? Power located in transmission: power located in csi? Probably will be constant

- HARQ off ok

Other considerations, add-ons:

- DMRS can be investigated/covered if necessary

- Energy Efficiency vs Spectral Efficiency (Reduce #Tx antennas at low load scenario?)

- 。。

Timeline

17-Dec: Initial progress, feasibility study

9-Jan: Checking on results, and interesting findings to present in the reports

14-Jan: Check on the milestone report (22-Jan due)

Further work afterwards

Gather result by 1st May,

To do: