Impact of CSI Feedback Granularity and PMI Codebook Size on Downlink MIMO Throughput in 5G NR

Overall Result presenting Structure

- Model Validation against Industry References <Result #1, done>

  - How my model behaves comparing to other companies’ results

    - 30, 70% throughput’s SNR deviation

    - Overall curve performance: saturation, turning point

  - Explain why it is impossible to be 100% coincide with other companies’ results

    - Simulation derivation spreadsheet from 3gpp meetings.

    - The spec 38101.4 includes impairment margin (avg.) -> 2 dB worth than the pure simulation (UE, real-life consideration) -> mention in report

    - Spreadsheet one does not include, pure simulation. If do TDL in spec, there is a impairment in the spec, so may not be coincide with the curve. {Discuss}

      - Linear average of dB value is not avg. of underpinning linear number two companies 2 3 -> mean 2.5 -> act. >2.5

  - Why this specific configuration AAV 1Y

- General results presentation, showing some trivial findings from my simulation

  - How 8 layers perform (2 CWs) <Result #2, need to rerun again>

  - How harq on/off selection impact the overall curve. <Result #3, done>

- CSI-related results:

  - Low speed scenario 10Hz, 4 Layers <Result #4>:

    - Impact of codebook resolution (Type, Model) and wide->subband

  - High speed scenario, 4 Layers<Result #5>:

    - Impact of CSI period to the throughput and HARQ retransmission rate

    - Snr-csi period-throughput (mbps) 3D plot <Result #6>

  - (If have time) How the findings change for 2/4/8 layers.

  - Overall csi feedback cost vs throughput benefit comparison

    - For a fix representative SNR point

CSI Benchmark Config. (38.101 -4 section 6 CSI reporting requirements)

6.2.1.1.1.1

- CSI periodicity: 10 slots, 5 offset

- CQI format: Wideband

- PMI format: Wideband

- Codebook type: Type I Single-Panel

- Codebook mode: Mode 1

- Subband size: 8 RB (only relevant when subband reporting is enabled)

Also, as a reference (yet to check) :

simParameters.CSIRS.CSIRSType = 'nzp';

simParameters.CSIRS.SubcarrierLocations = 0;

simParameters.CSIRS.Density = 'one';

simParameters.UEProcessingDelay = 7;

simParameters.BSProcessingDelay = 1;

CSI Scanning